MB1343-A (ST25TV_eSEAL)

PCB Manufacturing Specification

Author: D. ROMAN

Sept. 12th 2017

# Modification History

NOTE: Change bars at the edge of each page show changes between revisions of document.

Please note that Section 1 Modification History on page 2 should only be used as a guide to the document changes and the whole document should be referenced for complete changes.

Revision A

Created by Denis ROMAN September-12-2017

# Contents

1	Modification History	2

2	Contents	3

3	PCB Manufacturing Specifications	4

3.1	Stackup description	4

3.2	PCB overview	5

# PCB Manufacturing Specifications

## Stackup description

Feel the table or provide a stackup document from Layout

|  | Document type | File name (proposed) | Status |
| --- | --- | --- | --- |
| copper | CompoNent Side layer | ST25TV-eSEAL_MB1343-A.GTL |  |
| copper | SolDer Side layer | ST25TV-eSEAL_MB1343-A.GBL |  |
| Resist | Component Resist Solder | ST25TV-eSEAL_MB1343-A.GTS |  |
| Resist | Bottom Resist Solder | ST25TV-eSEAL_MB1343-A.GBS |  |
| Silkscreen | Top Silkscreen | ST25TV-eSEAL_MB1343-A.GTO |  |
| Silkscreen | Bottom Silkscreen | NA |  |
| Drill | Drilling drawing | ST25TV-eSEAL_MB1343-A-DrillDrawing.GD1 |  |
| Drill | Drilling file / Drill tooling |  |  |
| Paste | Overlay Top Paste | ST25TV-eSEAL_MB1343-A.GTP |  |
| Paste | Overlay Bottom Paste | ST25TV-eSEAL_MB1343-A.GBP |  |

## PCB overview

Description of the PCB characteristics

| Track width | 0.25mm |
| --- | --- |
| Minimum spacing between tracks/pads | 0.15mm |
| Bare board test required (Single/Double sided) Double sided | Double sided |
| Silkscreen colour (White, Blue …) | Blue |
| Does the board have surface mount components (Yes/No) Yes, both surfaces | Yes, Both Sides |
| Plating type: Chemical Gold plating (See note), Selective Tin/lead GOLD, Tin HAL, Chemical Tin | Chemical Gold |
| Solder resist type , colour (Green, Blue, Red, White, …) | White |
| PCB material FR4 HTG xxx | HTG 135° |
| Copper weight (Outer) 1oz | 17.5 µm or 0.5Oz |
| Copper weight (Inner) 0.5oz | 17.5 µm or 0.5Oz |
| PCB Thickness (not including resist) xxmm +/- 0.15mm | 1.2mm +/- 10% |
| PCB Thickness (including resist) xxmm +/- 0.20mm | 1.2mm +/- 0.20mm |
| Number of PCB layers | 2 |
| PCB Board Size | 62mm x 62 mm |
| Panel Size (if required) |  |
| RoHS compliant | Yes |

Notes

1 The following standard PCB manufacturing edits are accepted: teardrop addition, non-functional pad removal. No addition of copper in waste areas because of copper antenna

2 If gold plating required use selected nickel gold plating, a flash of nickel at 5 micron and gold at 0.1micron.

3 Silkscreen clipping is acceptable as long as no pin 1 markers are removed.

4 Soldermask removal around vias drill holes is accepted.