MB895C Board

PCB Manufacturing Specification

# Modification History

Please note that Section 1 Modification History on page 2, should only be used as a guide to the document changes and the whole document should be referenced for complete changes.

Revision

Release A

Initial version

Release B

SCH and layout update

Release C

layout update

# Contents

1	Modification History	2

2	Contents	3

3	PCB Manufacturing Specifications	4

3.1	Stackup description	4

3.2	PCB overview	4

4	PCB Milling diagram	5

5	PCB Drill schedule	5

# PCB Manufacturing Specifications

## Stackup description

Stackup is a follow:

Artwork 1: Top Silkscreen

Artwork 2: Top solder mask

Artwork 3: Top Layer (signal1)

Artwork 4: Bottom Layer (signal2)

Artwork 5: Bottom solder mask

Artwork 6: Bottom Silkscreen

The PCB should be manufactured to the BS9621 PCB build standard and to the STMicroelectronics specification 7018869.

## PCB overview

| Minimum Track width | 0.18mm / 7mil |
| --- | --- |
| Minimum spacing between tracks/pads | 0.20mm / 8 mil |
| Minimum hole size | 0.20mm /8mil |
| Bare board test required (Single/Double sided) Double sided | Double sided |
| Silkscreen colour | White |
| Does the board have surface mount components (Yes/No) Yes, both surfaces | No, both surfaces |
| Plating type: Chemical Gold plating (See note), Selective Tin/lead GOLD, Tin HAL, Chemical Tin | lead GOLD |
| Solder resist type , colour | Probimer, colour green |
| PCB material FR4 HTG xxx | FR4HTG175° |
| Copper weight (Outer) | 35µm/1oz |
| Copper weight (Inner) | 0 |
| PCB Thickness (not including resist) 1.6mm +/- 0.15mm | 1.6mm +/- 0.15mm |
| PCB Thickness (including resist) 1.6mm +/- 0.20mm | 1.6mm +/- 0.20mm |
| PCB size | 84 x44 mm |

Notes

1 The following standard PCB manufacturing edits are accepted: teardrop addition, non-functional pad removal, addition of copper in waste areas

2 If gold plating required use selected nickel gold plating, a flash of nickel at 5 micron and gold at 0.1micron.

3 Silkscreen clipping is acceptable as long as no pin 1 markers are removed.

4 Soldermask removal around vias drill holes is accepted.

# PCB Milling  diagram and Drill schedule

Please refer to the following documents:

Drill drawing through holes.gbr

bottom solder mask.gbr