# STEVAL–LLL002M1: ALED1262 EVALUATION BOARD

# Setup

The testing procedure for the STEVAL–LLL002M1 requires:

- 1 x STEVAL–LLL002V1 evaluation kit (STEVAL-LLL002M1 and STEVAL-LLL002D1)

- 1 x DC Power supply – Rating:12–20V / 1.5A

- 1 x DC Power Supply – Rating:18V / 100mA

- PC with a free USB port

- 1 x ST-LINK/V2

- 1 x USB cable A type/Mini-USB B type

- 1 x SWIM Programing Cable to program STM8AF62xx

Software requirements:

- Firmware Hex file “STEVAL-LLL002M1.hex”

- Chip Programing Firmware File “

  - STEVAL-LLL002M1 - CS1.hex

  - STEVAL-LLL002M1 - CS2.hex

  - STEVAL-LLL002M1 - CS3.hex

  - STEVAL-LLL002M1 - CS4.hex

- A PC with the following characteristics:

  - Operative system Windows XP / Windows 7

  - Installed software: ST Visual Programmer STM8 (STVP-STM8)
It can be downloaded from http://www.st.com/ (code STVP-STM8)

# Programming the STEVAL–LLL002M1

- Download and install the ST Visual Programmer STM8 software.

- Using SWIM programmer cable connect the STEVAL–LLL002M1 board (J3) to the ST-Link/V2.

- Ensure all Jumpers are closed before powering up the board (J1, J4, J5, J6, J7 and J8).

- Power up board by applying DC voltage at CN1 with in the range of 12–20V.

NOTE: Power Supply should not be removed from CN1 till the complete programming of the board.

- Wait for the installation of the ST-LINK/V2 drivers. (STSW-LINK009)

NOTE: The ST-LINK/V2 drivers are also available on www.st.com.

- Launch ST Visual Programmer.

- In Configure window select the following and press OK.

- Hardware  ST-LINK

- PORT  USB

- Programing MODE  SWIM

- DEVICE  STM8AF6266

- Open STEVAL-LLL002M1 - CS1.hex file in a program memory window.

- Select AFR6 as Port B5 Alternate Function = I2C_SDA, Port B4 Alternate Function = I2C_SCL in an option byte window.

- Click on Program  All tabs (on active sensors if any).

- After clicking wait till the option byte successfully verified.

NOTE: In case of any error check the connections and re-initiate.

- Connect 18V on the STEVAL-LLL002M1 board connections:

  - 18V – Pin 1 of J2

  - GND – Pin 5 of J2

- Close the ST Visual Programmer.

- Remove 18V from Pin 1 of J2.

- Re-launch ST Visual Programmer.

- Open STEVAL-LLL002M1 – CS2.hex file in a program memory window.

- Click on Program  Current tab.

- Wait till the program memory get loaded successfully.

- Connect 18V on the STEVAL-LLL002M1 board connections:

  - 18V – Pin 2 of J2

  - GND – Pin 5 of J2

- Close the ST Visual Programmer.

- Remove 18V from Pin 2 of J2.

- Re-launch ST Visual Programmer.

- Open STEVAL-LLL002M1 – CS3.hex file in a program memory window.

- Click on Program  Current tab.

- Connect 18V on the STEVAL-LLL002M1 board connections:

  - 18V – Pin 3 of J2

  - GND – Pin 5 of J2

- Wait till the program memory get loaded successfully.

- Close the ST Visual Programmer.

- Remove 18V from Pin 3 of J2.

- Re-launch ST Visual Programmer.

- Open STEVAL-LLL002M1 – CS4.hex file in a program memory window.

- Click on Program  Current tab.

- Connect 18V on the STEVAL-LLL002M1 board connections:

  - 18V – Pin 4 of J2

  - GND – Pin 5 of J2

- Wait till the program memory get loaded successfully.

- Close the ST Visual Programmer.

- Remove 18V and GND from Pin 4 and Pin 5 of J2.

- Re-launch ST Visual Programmer.

- Open STEVAL-LLL002M1.hex file in a program memory window.

- Click on Program  Current tab.

- Wait till the program memory get loaded successfully.

- Close the ST Visual Programmer.

- The final firmware has been uploaded in the MCU. Dot-sequence rail pattern should run on to the board.

# Testing of the evaluation board

- On powering up the board at CN1 (12–20V), the board should display a dot sequence rail pattern.

- STEVAL-LLL002M1 is equipped with 2 push–buttons (S1 and S2) and 2 potentiometers (R15 and R16).

  - Press S2 and all LED’s should glow.

  - Moving R16 the brightness should change. The brightness should increase on rotating R16 clock-wise.

  - Press S1 and dot sequence pattern should come back.

  - Moving R15 the speed of the dot-sequence rail should change. The speed should increase on rotating R15 clockwise.

  - Press S1 again, then the board enter in error detection mode.

  - Remove D48  D44 should glow.

  - Remove D49  D45 should glow.

  - Remove D50  D46 should glow.

  - Remove D51  D47 should glow.

- Disconnect the power supply from the board.

- Connect the USB–UART Bridge (STEVAL–LLL002D1) with the STEVAL–LLL002M1 board.

- Using USB cable connect the USB–UART Bridge (STEVAL–LLL002D1) with the PC.

- Re-connect the power supply.

  - STM32 Virtual COM Port driver will be installed on the PC.

- Launch Graphic user interface (GUI) for the STEVAL–LLL002V1 kit and click on connect.

- Port COMxx is Open will appear in the message section.

- Click on Basic Mode.

- Click on   FWD icon. The LED pattern/demo on the STEVAL–LLL002M1 board should be changed.

- Click on BCK icon. The LED pattern/demo on the STEVAL–LLL002M1 board should come back to the previous demo.

- Close the GUI and disconnect the power supply from the STEVAL–LLL002M1 board.