IBIS Quality Report

| Company: | STMicroelectronics |
| --- | --- |
| IBIS file name | rhfac2525_3v3.ibs |
| IBIS Version: | 4.0 |

Contents

1. Modeling	3

1.1	Component description	3

1.2	Modeling conditions	3

1.3	Circuit for data extraction	5

2. IBISCHK6 Check	6

2.1	Result Check by IBISCHK6	6

3. FUNCTIONAL Check	7

3.1	Functional verification	7

4. Extra information	8

# 1. Modeling

IBIS (I/O, Buffer, Information, Specification) provide a standardized way, officially EIA standard 656-A-1999 and IEC 62014-1, to model behaviorally a digital component input, output and I/O buffers.

## Component description

| Component name | Technology | Component description |
| --- | --- | --- |
| RHFAC2525 | CMOS | The RHFAC2525 device is a Rad-Hard 1-to-8 minimum skew Clock Driver |

## Modeling conditions

| Simulator used | AMS 2010.2b (Mentor Graphics) |
| --- | --- |

Figure 1: IBIS model generic structure

| conditions | Typical | Minimum | Maximum |
| --- | --- | --- | --- |
| Temperature [C] | 25 | -55 | 125 |
| Voltage Supply [Volt] | 3.3 | 3.0 | 3.6 |
| Process setting | nom | weak | strong |

| Model names (of Component) | Model Type | C_comp (typ, min, max) |
| --- | --- | --- |
| mod_ck | Input | 2.119pF (typ), 2.068pF(min) , 2.200pF(max) |
| mod_out | Output | 11.120pF (typ), 10.610pF (min) , 11.610pF(max) |

| Model names (of Component) | Threshold and Vmeas | Timing parameters (if used) |
| --- | --- | --- |
| mod_ck | Vil=0.660V , Vih=2.640V |  |
| mod_out | Vmeas=1.650V | Cref=50pF , Rref=500 |

| Package | Description |
| --- | --- |
| 14L Ceramic Flat | 14 leads ceramic package |

## Circuit for data extraction

The I-V data are extracted by simulations using the simulation setup shown in figure 2 below. This model is an I/O model, other model type derived from this structure. For more accurate modeling, certain combinations of V-T tables are recommended (with exception of Input-only model types) using the simulation setup shown in figure 3, with load conditions specified.

Figure 2: Simulation Setup to extract I/V data from I/O model type

Figure 3: Simulation Setup to extract V/T data from I/O model type

(see also Table 1)

| V/T data condition extractions | Load conditions |
| --- | --- |
| Rising waveform | R_fix=50 Ohm, V_fix= 0 V |
| Rising waveform | R_fix=50 Ohm, V_fix= 3.3 V |
| Falling waveform | R_fix=50 Ohm, V_fix= 3.3 V |
| Falling waveform | R_fix=50 Ohm, V_fix= 0 V |

Table 1: V/T curve extraction load conditions

# 2. IBISCHK6 Check

The created IBIS model must be checked using IBISCHK6 parser to ensure that the syntax is correct. The result of the check is showed in the next section with some comments (optional).

## Result Check by IBISCHK6

IBISCHK6 V6.0.1

Checking rhfac2525_3v3.ibs for IBIS 4.2 Compatibility...

Errors  : 0

File Passed

Adding comments about the Warning or Note:

The output check contains zero Notes, zero Warnings and zero Errors.

# 3. FUNCTIONAL Check

The created IBIS model must be compared with the Original Buffer circuit. The signal outputs, in the same load conditions (                    Figure 4), must match. These output comparisons are presented in TYP, MIN and MAX condition. This section cannot be defined for Input and Terminator model type because they are input-only model types.

| How well results are matched? | Put “X” into the right filed |
| --- | --- |
| Curves shape match correctly, but there is a little time translation. |  |
| Curves shape match correctly, but there is a mismatch into the Overshoot and/or Undershoot regions. |  |
| Curves match well. | X |

## Functional verification

Circuit used for output comparison results is illustrated in figure 4.

Figure 4: Circuit used for functional check

Figure 5: IBIS vs Eldo comparison results of “mod_out” Model

Output Comparisons:

Adding comments about the comparison:

# 4. Extra information

This section can contain other extra information, to explain some other features of peculiar IBIS model

| Other specifications | description |
| --- | --- |