IBIS Quality Report

| Company: | STMicroelectronics |
| --- | --- |
| IBIS file name | m95m04dr_tssop8_3d3v.ibs |
| IBIS Version: | 4.0 |

Contents

1. Modeling	3

1.1	Component description	3

1.2	Modeling conditions	3

1.3	Circuit for data extraction	5

2. IBISCHK6 Check	6

2.1	Result Check by IBISCHK6	6

3. FUNCTIONAL Check	7

3.1	Functional verification	7

4. Extra information	8

# 1. Modeling

IBIS (I/O, Buffer, Information, Specification) provide a standardized way, officially EIA standard 656-A-1999 and IEC 62014-1, to model behaviorally a digital component input, output and I/O buffers.

## Component description

| Component name | Technology | Component description |
| --- | --- | --- |
| M95M04DR | CMOS | The M95M04DR is a 4-Mbit serial SPI bus EEPROM memory with high-speed clock. This IBIS model is in TSSOP8 package. |

## Modeling conditions

| Simulator used | AMS 2015.4 (Mentor Graphics) |
| --- | --- |

Figure 1: IBIS model generic structure

| conditions | Typical | Minimum | Maximum |
| --- | --- | --- | --- |
| Temperature [C] | 25 | -40 | 85 |
| Voltage Supply [Volt] | 3.3 | 3.0 | 3.6 |
| Process setting | nom | weak | strong |

| Model names (of Component) | Model Type | C_comp (typ, min, max) |
| --- | --- | --- |
| mod_s | Input | 3.063pF (typ), 2.925pF(min) , 3.207pF(max) |
| mod_q | I/O | 4.636pF (typ), 4.427pF (min) , 4.855pF(max) |
| mod_w | Input | 3.063pF (typ), 2.925pF(min) , 3.207pF(max) |
| mod_h | Input | 3.063pF (typ), 2.925pF(min) , 3.207pF(max) |

| Model names (of Component) | Threshold and Vmeas | Timing parameters (if used) |
| --- | --- | --- |
| mod_s/w/h | Vil=1.000V , Vih=2.300V |  |
| mod_q | Vmeas=1.650V | Cref=30pF |

| Package | Description |
| --- | --- |
| TSSOP8 | 8 pin thin shrink small outline |

## Circuit for data extraction

The I-V data are extracted by simulations using the simulation setup shown in figure 2 below. This model is an I/O model, other model type derived from this structure. For more accurate modeling, certain combinations of V-T tables are recommended (with exception of Input-only model types) using the simulation setup shown in figure 3, with load conditions specified.

Figure 2: Simulation Setup to extract I/V data from I/O model type

Figure 3: Simulation Setup to extract V/T data from I/O model type

(see also Table 1)

| V/T data condition extractions | Load conditions |
| --- | --- |
| Rising waveform | R_fix=50 Ohm, V_fix= 0 V |
| Rising waveform | R_fix=50 Ohm, V_fix= 3.3 V |
| Falling waveform | R_fix=50 Ohm, V_fix= 3.3 V |
| Falling waveform | R_fix=50 Ohm, V_fix= 0 V |

Table 1: V/T curve extraction load conditions

# 2. IBISCHK6 Check

The created IBIS model must be checked using IBISCHK6 parser to ensure that the syntax is correct. The result of the check is showed in the next section with some comments (optional).

## Result Check by IBISCHK6

IBISCHK6 V6.0.1

Checking m95m04dr_tssop8_3d3v.ibs for IBIS 4.2 Compatibility...

NOTE (line  300) - Pulldown Typical data is non-monotonic

NOTE (line  301) - Pulldown Minimum data is non-monotonic

NOTE (line  302) - Pulldown Maximum data is non-monotonic

NOTE (line  404) - Pullup Maximum data is non-monotonic

NOTE (line  405) - Pullup Typical data is non-monotonic

NOTE (line  406) - Pullup Minimum data is non-monotonic

Errors  : 0

File Passed

Adding comments about the Warning or Note:

The output check contains some Note about non-monotonic data of I-V curves, but they are not indicative of problems inside the model.

# 3. FUNCTIONAL Check

The created IBIS model must be compared with the Original Buffer circuit. The signal outputs, in the same load conditions (                     Figure 4), must match. These output comparisons are presented in TYP, MIN and MAX condition. This section cannot be defined for Input and Terminator model type, because they are input-only model types.

| How well results are matched? | Put “X” into the right filed |
| --- | --- |
| Curves shape match correctly, but there is a little time translation. |  |
| Curves shape match correctly, but there is a mismatch into the Overshoot and/or Undershoot regions. |  |
| Curves match well. | X |

## Functional verification

Circuit used for output comparison results is illustrated in figure 4.

Figure 4: Circuit used for functional check

Figure 5: IBIS vs Eldo comparison results of “mod_q” Model

Output Comparisons:

Adding comments about the comparison:

# 4. Extra information

This section can contain other extra informations, to explain some other features of peculiar IBIS model

| Other specifications | description |
| --- | --- |